# Phase 3 Vision Pipeline: Progress & Experiments Report

# 1. Pipeline Overview

The Phase 3 architecture extends the existing local RAG pipeline to support pre-recorded video. It acts as a "Video-to-Text RAG Pipeline" running locally, rigorously tested under simulated mobile constraints (CPU-only inference, sub-4GB RAM limits).

Key highlights include the Shared Architecture: using a singular runtime engine for both VLM caption generation and LLM response generation, achieving massive memory efficiency suitable for Android deployment.

# 2. Experiment Findings (A1 to A4)

## Experiment A1: Quantization vs. Accuracy

Evaluates the quality drop when shrinking the model for mobile deployment via GGUF quantization.

| Model Variant | Disk Size (MB) | RAM (MB) | Avg Latency (s) | Cosine Similarity |
| --- | --- | --- | --- | --- |
| 256M-Q8_0 | 266 | 69 | 14.0 | 0.809 |
| 500M-Q8_0 | 520 | 101 | 15.7 | 0.864 |
| 500M-F16 (Baseline) | 973 | 99 | 16.9 | 1.0 |

Finding: The 500M-Q8_0 quantization halved the disk space required while maintaining a 0.864 similarity score, proving the model can fit on a smartphone without devastating accuracy loss.

## Experiment A2: Frame Sampling Rate vs. Recall

Finds the optimal extraction interval to balance CPU processing time and contextual recall.

| Interval | Frames Sampled | Processing Time (s) | Hit@3 | Hit@5 (Recall) | MRR |
| --- | --- | --- | --- | --- | --- |
| 10.0s | 1 | 30.1 | 0.67 | 0.67 | 0.67 |
| 5.0s | 2 | 39.9 | 1.00 | 1.00 | 1.00 |
| 3.0s | 4 | 74.6 | 1.00 | 1.00 | 1.00 |
| 1.0s | 10 | 186.3 | 1.00 | 1.00 | 1.00 |

Finding: 1 frame every 5 seconds is optimal. It successfully retrieved 100% (Hit@5 = 1.0) of required information while cutting processing time by nearly 80% compared to 1-second sampling.

## Experiment A3: Caption Style (Short vs Long)

Tests whether brief 1-sentence captions embed and retrieve more accurately than verbose descriptions. The pipeline prompt orchestrator is successfully configured to support multiple styles. Final quantitative dataset mapping is pending baseline approvals.

## Experiment A4: Sustained Processing / Thermal Soak Test

Validates stability when running continuous mobile-simulated CPU workloads.

• Duration: 30.21 minutes
• Total Frames Processed: 136
• Peak RAM: 752 MB (Limit: 4000 MB)
• Peak CPU: 79.0%
• Average Caption Latency: 13.32s
• Latency Drift: 1.2% (Proves zero significant thermal throttling)

# 3. Future Plans & Next Steps

## Phase 3 Remaining Experiments (A5 - A7)

Before freezing the architecture for mobile deployment, the remaining experiments will be executed and recorded:

1. Experiment A5 (Cross-Pipeline Integration): Mix 50 video memories into the existing text RAG database to verify no degradation in keyword extraction and semantic search scores.
2. Experiment A6 (Model Head-to-Head): Execute standard retrieval benchmarks comparing our Gemma 4 E2B implementation against dedicated VLMs (e.g., Moondream2 / PaliGemma) to conclusively prove the 'Shared Architecture' efficiency gains.
3. Experiment A7 (Optimizations): Refine multi-frame context aggregation and prompt tuning to recover any minor percentage drops caused by quantization.

## Mobile Transition Strategy

Upon successful conclusion and documentation of A5-A7, we will migrate directly to Android:
• The validated LiteRT-LM models will be ported to the Android app.
• The SQLite database structure will map directly 1:1 onto Android Room.
• The UI will transition from Streamlit to native Kotlin, utilizing real on-device storage.